MODULE 2 – SIGNAL PREPROCESSING

Three-Week Work Plan with Validated Formulae and Parameter Definitions

Scope: Process complex S21 from valid Touchstone .s2p files and prepare calibrated time-domain data for Module 3 DAS/DMAS reconstruction. Conditional steps must be skipped and reported when the required data are unavailable.

| Timeline | Step and purpose | Reason | Formulae and definitions | Expected output |
| --- | --- | --- | --- | --- |
| Week 1 | 1. Import and inspect the Touchstone file | Identify the frequency unit, data format and reference impedance; extract only S21. | fHz=fGHz×109 fHz=fMHz×106 f: frequency; S21: complex transmission coefficient from Port 1 to Port 2. | Header information and extracted S21. |
|  | 2. Convert S21 to complex linear form | Preserve magnitude and phase. Do not directly average, filter or subtract dB values. | Ak=10Dk/20 ϕk=ϕdegkπ180 S21k=Akcosϕk+jsinϕk S21k=Rk+jIk D[k]: magnitude in dB; A[k]: linear magnitude; φ: phase in radians; R and I: real and imaginary parts; j² = −1. | Raw complex S21. |
|  | 3. Validate and sort samples | Prevent invalid values, duplicated frequencies and incorrect ordering from affecting later processing. | Nf=NS21 fk+1>fk Nf and NS21: lengths of the frequency and S21 vectors; k: frequency index. | Validated, ascending frequency and S21 vectors. |
|  | 4. Check uniform spacing and interpolate only if required | A conventional IFFT requires uniformly spaced frequency samples; matched files require the same grid. | Δfk=fk+1-fk fuk=fmin+kΔfu Δfu=fmax-fminNu-1 Ruk=Interpf,R,fuk Iuk=Interpf,I,fuk S21,uk=Ruk+jIuk Nu: number of uniform-grid samples; Δfu: uniform spacing; Interp: documented interpolation method, initially linear. | Uniform frequency vector and complex S21. |
|  | 5. Generate magnitude and phase plots | Inspect the original signal and retain both wrapped and continuous phase representations. | S21,dBk=20log10S21,uk ϕwk=atan2Iuk,Ruk ϕuk=unwrapϕwk φw: wrapped phase; φu: unwrapped phase. Unwrapping corrects representation jumps; it does not denoise S21. | Magnitude, real, imaginary, wrapped-phase and unwrapped-phase plots. |
| Week 2 | 6. Detect isolated invalid samples, only when present | Prevent isolated acquisition errors from generating artificial broadband artefacts. | mk=median{xr:r∈Wk} MADk=median{xr-mk:r∈Wk} σk=1.4826 MADk xk-mk>λσk Wk: local window; mk: local median; MAD: median absolute deviation; λ: reported threshold, commonly 3. | Corrected complex S21 and a log of corrected samples. |
|  | 7. Average repeated sweeps, when available | Reduce uncorrelated random variation in repeated measurements made under identical conditions. | S21k=1Mm=1MS21mk M: number of repeated sweeps; m: sweep index. Do not average different antenna positions or phantom conditions. | Complex-averaged S21. |
|  | 8. Apply and compare mild noise filters | Reduce point-to-point noise without removing weak target-related variations. | Rfk=1Lr=-qqRk+r Ifk=1Lr=-qqIk+r L=2q+1 S21,fk=Rfk+jIfk L: odd filter length; q: neighbours on either side. Use documented edge handling and identical settings for all comparable files. | Filtered complex S21 and filter-comparison plots. |
|  | 9. Quantify filtering change | Check that filtering has not excessively altered the complex signal. | NRMSE=k=0N-1S21,fk-S21k2k=0N-1S21k2 NRMSE: normalized RMS change. It measures alteration, not reconstruction quality by itself. | Selected filter, parameters, NRMSE and validation plots. |
| Week 3 | 10. Apply a Hamming window | Taper finite-band edges and reduce time-domain ringing and sidelobes. Preserve the unwindowed filtered signal. | wHk=0.54-0.46cos2πkN-1 S21,Hk=S21,fkwHk N: number of uniform frequency samples; wH: Hamming coefficient; k = 0,…,N−1. | Filtered and Hamming-windowed complex S21. |
|  | 11. Window target and reference identically | Maintain a valid matched comparison before subtraction. | Sp,T,Hk=Sp,T,fkwHk Sp,R,Hk=Sp,R,fkwHk p: Tx–Rx channel; T: target/tumor-present; R: matched reference/tumor-free; H: Hamming-windowed. | Matched windowed target and reference signals. |
|  | 12. Convert to the complex time-domain response | Provide delay-domain signals for comparison and DAS/DMAS reconstruction. | s21,Hn=1Nk=0N-1S21,Hkej2πkn/N Δt=1NΔf tn=nΔt Tperiod=1Δf δt≈1B B=fmax-fmin=N-1Δf n: time index; Δt: time-sample interval; Tperiod: represented time span; B: retained bandwidth; δt: approximate time resolution. Direct bandpass IFFT yields a complex equivalent response. | Time vector and time-domain signals with and without Hamming windowing. |
|  | 13. Perform matched reference subtraction | Suppress the system and tumor-free/background response, retaining target-related change. | Sp,diff,Hk=Sp,T,Hk-Sp,R,Hk sp,diffn=sp,Tn-sp,Rn IFFTST-SR=IFFTST-IFFTSR Target and reference must have identical calibration, geometry, frequency grid, window and preprocessing. | Reference-subtracted complex signal. |
|  | 14. Remove common channel clutter, only when geometry permits | Estimate and subtract response common to a group of comparable antenna channels. | μGn=1Pp=1Psp,diffn sp,cleann=sp,diffn-μGn G: selected comparable channel group; P: number of channels in G; μG: group-mean clutter estimate. | Clutter-reduced channel signals. |
|  | 15. Generate an optional globally normalized copy | Provide a common scale while preserving relative amplitude between channels. | α=maxp,nsp,cleann sp,cleann=sp,cleannα α: one maximum calculated across every selected channel and time sample. Do not normalize each channel independently. | Original-scale and globally normalized copies. |
|  | 16. Validate and export for Module 3 | Preserve traceability and provide all signals and geometry required for image reconstruction. | Check equal dimensions, uniform spacing, matching metadata, complex values and absence of NaN/Inf. | .mat file, optional .csv, code, plots, parameters, Tx/Rx coordinates and validation report. |

# Required Module 3 Handover

frequency_Hz and time_s

S21_raw, S21_filtered and S21_Hamming

S21_reference_subtracted and S21_clutter_removed (where applicable)

Tx_coordinates, Rx_coordinates and measurement_labels

processing_parameters, commented code, plots and validation report

# Technical Conditions

Phase unwrapping is retained for phase inspection and delay analysis; it is not a denoising operation and does not replace the original complex S21.

Filtering, spike correction, repeated-sweep averaging and channel-average clutter removal are conditional. Their use and parameters must be justified and reported.

The target and reference must use identical calibration, antenna geometry, frequency grid, window and preprocessing.

Repeated-sweep averaging and antenna-channel averaging are different operations and must not be interchanged.

Use one global normalization factor. Independent channel-wise normalization may destroy relative amplitude information needed for localization.

A direct IFFT of measurements starting at a nonzero frequency yields a complex bandpass-equivalent response. A real physical waveform requires appropriate conjugate-symmetric spectral construction.

# Peer-Reviewed Method Support

1. Blanco-Angulo, C., et al., “Non-Invasive Microwave-Based Imaging System for Early Detection of Breast Tumours,” Biosensors, 12, 752, 2022. DOI: 10.3390/bios12090752. Reports Hamming-window preprocessing, time-domain conversion, reference subtraction, averaging-based artefact removal and IDAS reconstruction.

2. Hammouch, N., et al., “A Low-Cost UWB Microwave Imaging System for Early-Stage Breast Cancer Detection,” Multimedia Tools and Applications, 84, 17329–17360, 2025. DOI: 10.1007/s11042-024-19761-0. Provides IFFT, tumor-present minus tumor-free calibration and antenna-average clutter-removal equations.